| Model | 2100HS | 2200HS | 2300HS | 2600HS | 210KHS |
| --- | --- | --- | --- | --- | --- |
| Rated Power |  |  |  |  |  |
| VA/W | 1KVA/0.7kW | 2KVA/1.4kW | 3KVA/2.1kW | 6KVA/4.2kW | 10KVA/7kW |
| Input |  |  |  |  |  |
| Nominal Voltage | 110VAC/220VAC |  |  | 220VAC |  |
| Voltage Range | 60～138VAC/115～300VAC |  |  | 176～276VAC |  |
| Frequency Range | 46～54Hz(default),user adjustable within 40～60Hz |  |  |  |  |
| PowerFactor | 0.97 |  |  |  |  |
| Output |  |  |  |  |  |
| Voltage | 110VAC/220VAC±2% |  |  | 220VAC±1% |  |
| Frequency | 46-54Hz±0.2Hz(Line mode) 50Hz±0.2Hz(battery mode) |  |  |  |  |
| Waveform | sine wave |  |  |  |  |
| Distortion | ≤2% under linear load | ≤2% under linear load |  | ≤2% under linear load |  |
| PowerFactor | 0.7 |  |  |  |  |
| Overload capability | 108%~150%for30secends.,±150%for300ms |  |  |  |  |
| Current Crest Ratio | 03:01 |  |  |  |  |
| Battery |  |  |  |  |  |
| Type | Sealed, Maintenance free, Lead-Acid |  |  |  |  |
| Recharge Time | 8hrs to90%of capacity after full load discharge |  |  |  |  |
| Bypass |  |  |  |  |  |
| Automatic Transfer | on overload and UPS failure |  |  |  |  |
| Before UPS power-on | Default "NO", user adjustable to "YES" |  |  |  |  |
| Transfer time | 0ms;less than 4 ms from inverter to bypass and vice versa |  |  |  |  |
| LED/LCD Indicators |  |  |  |  |  |
| status | load level/Battery level, Battery, Utility power, Inverter, Bypass, Overload, Fault |  |  |  |  |
| Audible Alarm | Bypass, ON battery mode, Battery low, Overload, Fault |  |  |  |  |
| Communications |  |  |  |  |  |
| DB-9Port | RS-232 Serial Interface |  |  |  |  |
| Intelligent Slot | Options;Webpowet,As400 Card or WinPower CMC |  |  |  |  |
| Weight |  |  |  |  |  |
| Net (Kg) | 14/7 | 34/13.5 | 34.5/14 | 90/35 | 93/38 |
| Dimensions |  |  |  |  |  |
| W×D×H (mm) | 145*400*220 | 192*460*340 |  | 260*570*717 |  |